# Mechanical Ventilation Management

marp: true

theme: default

paginate: true

backgroundColor: #fff

# Mechanical Ventilation Management

## Comprehensive Guide to Ventilator Management

Presenter: [Name], RRT, [Credentials]

Date: [Date]

Department: Respiratory Care

# Learning Objectives

By the end of this session, participants will be able to:

Describe the components of a mechanical ventilator and breathing circuit

Select appropriate ventilator modes based on patient condition

Apply lung-protective ventilation strategies

Interpret ventilator graphics and waveforms

Troubleshoot common ventilator alarms and patient-ventilator asynchrony

Develop and implement ventilator weaning protocols

# Indications for Mechanical Ventilation

## When to Intubate and Ventilate

Primary Indications:

Apnea or Respiratory Arrest

No spontaneous breathing

Immediate intervention required

Acute Respiratory Failure

Hypoxemic: PaO2 <60 mmHg on high FIO2

Hypercapnic: pH <7.30 with elevated PaCO2

Increased work of breathing

Airway Protection

Altered mental status (GCS ≤8)

Inability to protect airway

Risk of aspiration

Prophylactic/Perioperative

Major surgery

Hemodynamic instability

Severe metabolic acidosis

# Ventilator Components

## The Mechanical Ventilator System

Control System:

Microprocessor control

User interface

Monitoring displays

Alarm systems

Pneumatic System:

Gas source (wall oxygen, medical air)

Pressure regulators

Flow controllers

Valves

Breathing Circuit:

Inspiratory limb

Expiratory limb

Y-connector

Humidifier

HME or heated humidifier

Patient Interface:

Endotracheal tube

Tracheostomy tube

Mask (for NIV)

# Ventilator Modes: Overview

## Classification by Control Variable

Volume Control (VC)

Delivers set tidal volume

Pressure varies with compliance/resistance

Guaranteed minute ventilation

Examples: VC-AC, VC-SIMV

Pressure Control (PC)

Delivers set inspiratory pressure

Volume varies with compliance/resistance

Decelerating flow pattern

Examples: PC-AC, PC-SIMV, PRVC

Pressure Support (PS)

Patient-triggered, pressure-limited

Flow-cycled

Used for spontaneous breathing

# Ventilator Modes: Assist/Control (A/C)

## Volume Assist/Control

Characteristics:

All breaths are volume-controlled

Patient can trigger breaths

If no trigger, ventilator delivers at set rate

Every breath has same volume

Settings:

Tidal volume (VT)

Respiratory rate (RR)

FIO2

PEEP

Flow rate

Inspiratory time or I:E ratio

Advantages:

Guaranteed minute ventilation

Good for patients with no spontaneous effort

Disadvantages:

Can cause asynchrony if patient wants different pattern

Risk of barotrauma if patient fights ventilator

# Ventilator Modes: SIMV

## Synchronized Intermittent Mandatory Ventilation

Characteristics:

Set number of mandatory breaths (volume or pressure)

Patient can breathe spontaneously between mandatory breaths

Spontaneous breaths can be unsupported or pressure-supported

Settings:

Mandatory rate

Tidal volume (if VC) or pressure (if PC)

Pressure support for spontaneous breaths

FIO2, PEEP

Advantages:

Allows spontaneous breathing

May reduce sedation needs

Theoretically prevents respiratory muscle atrophy

Disadvantages:

Can increase work of breathing

May prolong weaning

Not proven superior to A/C

# Ventilator Modes: Pressure Support

## Spontaneous Mode with Support

Characteristics:

Patient triggers every breath

Set pressure support level

Flow-cycled (cycles off when flow drops to % of peak)

No set rate or tidal volume

Settings:

Pressure support level (cmH2O)

PEEP

FIO2

Trigger sensitivity

Flow cycle criteria

Advantages:

Patient controls rate and tidal volume

Comfortable for awake patients

Good for weaning

Disadvantages:

No guaranteed minute ventilation

Apnea alarm essential

Not for patients with weak drive

# Ventilator Modes: Advanced Modes

## Pressure Regulated Volume Control (PRVC/VC+)

Characteristics:

Pressure-controlled breaths

Volume-targeted

Automatically adjusts pressure to deliver set volume

Advantages:

Decelerating flow (better distribution)

Volume guarantee

Automatic compliance compensation

## Airway Pressure Release Ventilation (APRV)

Characteristics:

Two pressure levels (Phigh and Plow)

Spontaneous breathing allowed at both levels

Time-cycled release

Advantages:

Good oxygenation

Allows spontaneous breathing

Lower peak pressures

# Lung-Protective Ventilation

## ARDSNet Protocol Principles

Tidal Volume:

6 ml/kg ideal body weight

Range: 4-8 ml/kg

Calculate IBW:

Men: 50 + 2.3 × (height in inches - 60)

Women: 45.5 + 2.3 × (height in inches - 60)

Plateau Pressure:

Goal: <30 cmH2O

If >30: decrease VT to minimum 4 ml/kg

PEEP:

Start at 5 cmH2O

Titrate using PEEP/FIO2 tables

Balance oxygenation vs. hemodynamics

Permissive Hypercapnia:

Accept PaCO2 up to 60 if pH ≥7.25

Don't increase VT to normalize CO2

# Initial Ventilator Setup

## Standard Starting Points

Volume A/C Mode:

Parameter | Setting | Notes

|-----------|---------|-------|

VT | 6-8 ml/kg IBW | Use IBW, not actual weight

Rate | 12-16/min | Adjust for pH/CO2

FIO2 | 100% | Titrate down quickly

PEEP | 5 cmH2O | Minimum to prevent atelectasis

Flow | 60 L/min | Adequate for most adults

I:E ratio | 1:2 to 1:3 | Avoid inverse ratio

Post-Intubation:

Check bilateral breath sounds

Verify ET tube position (depth marking)

Obtain chest X-ray

Check ABG in 30 minutes

# Ventilator Graphics

## Understanding Waveforms

Pressure-Time Waveform:

Shows pressure changes during breath

Peak pressure: Total resistance + compliance

Plateau pressure: Static compliance only

PEEP: Baseline pressure

Flow-Time Waveform:

Square wave (constant flow) - volume control

Decelerating wave - pressure control

Expiratory flow shows airway obstruction

Volume-Time Waveform:

Shows volume delivery and exhalation

Should return to zero

If not, air trapping present

# Ventilator Graphics Interpretation

## Common Patterns

Normal:

Smooth, consistent waveforms

Volume returns to baseline

Stable pressures

Airway Obstruction:

Prolonged expiratory flow

Scalloped expiratory flow waveform

Auto-PEEP

Asynchrony:

Pressure waveform notches (triggering issues)

Flow waveform abnormalities

Double triggering

Bronchodilator Response:

Decreased peak pressure

Improved expiratory flow

Reduced auto-PEEP

# Patient-Ventilator Asynchrony

## Types and Solutions

Trigger Asynchrony:

Problem: Patient effort doesn't trigger breath

Cause: Insensitive trigger, auto-PEEP

Solution: Increase sensitivity, reduce auto-PEEP

Flow Asynchrony:

Problem: Set flow doesn't match patient demand

Cause: Flow too low for patient's needs

Solution: Increase flow or switch to pressure control

Cycle Asynchrony:

Problem: Inspiration ends too early or late

Cause: Mismatch between neural and mechanical inspiration

Solution: Adjust flow cycle % or inspiratory time

Double Triggering:

Problem: Two breaths in rapid succession

Cause: Insufficient tidal volume, short inspiratory time

Solution: Increase VT or adjust flow

# Ventilator Alarms

## High Pressure Alarm

Causes:

Secretions in airway

Kinked ETT or circuit

Bronchospasm

Decreased lung compliance

Patient biting ETT

Water in circuit

Coughing or fighting ventilator

Response:

Check patient first - look, listen, feel

Suction if indicated

Check circuit for kinks or water

Auscultate for bronchospasm

Check ETT position and patency

Consider sedation if patient-ventilator dyssynchrony

If unresolved, disconnect and bag manually

# Ventilator Alarms (continued)

## Low Pressure/Volume Alarm

Causes:

Circuit disconnect

Cuff leak

Patient self-extubation

Leak in circuit

Response:

Check patient immediately

Verify ETT position (depth marking)

Check cuff pressure (20-30 cmH2O)

Check all connections

Listen for leak around ETT

If extubated, bag-valve-mask and call for help

## Low PEEP/CPAP Alarm

Causes:

Large leak in system

Circuit disconnect

Inadequate flow

Response:

Check circuit integrity

Verify all connections

Check for leaks around ETT

# Auto-PEEP (Intrinsic PEEP)

## Causes and Management

Definition:

Positive pressure remaining in alveoli at end-expiration due to incomplete emptying

Causes:

Airway obstruction (asthma, COPD)

High minute ventilation

Short expiratory time

High respiratory rate

Detection:

Expiratory flow doesn't return to zero

End-expiratory port occlusion shows pressure

Need to trigger against positive pressure

Management:

Increase expiratory time (decrease rate or inspiratory time)

Bronchodilators

Reduce minute ventilation (if appropriate)

Apply extrinsic PEEP (80% of auto-PEEP)

# Weaning from Mechanical Ventilation

## Daily Spontaneous Awakening and Breathing Trials

Readiness Criteria:

Resolution of acute illness

Hemodynamic stability (minimal or no vasopressors)

Oxygenation: PaO2/FIO2 >150-200, PEEP ≤5-8

Cough and airway reflexes present

No pending procedures requiring sedation

Spontaneous Breathing Trial (SBT):

CPAP 5 cmH2O, or

T-piece, or

Low pressure support (5-8 cmH2O)

Duration: 30-120 minutes

SBT Success Criteria:

RR <35

SpO2 >90%

HR <140 or <20% increase

No hemodynamic instability

No respiratory distress

# Weaning Parameters

## Objective Measures

Rapid Shallow Breathing Index (RSBI):

RR / VT (in liters)

<105: Likely to succeed

>105: Likely to fail

Negative Inspiratory Force (NIF):

<-20 to -30 cmH2O

Measures inspiratory muscle strength

Other Parameters:

Spontaneous VT >5 ml/kg

Vital capacity >10-15 ml/kg

Minute ventilation <10 L/min

Maximum voluntary ventilation >2× resting VE

Note: These predict success better than failure. Clinical assessment is most important.

# Extubation

## Procedure and Considerations

Pre-Extubation:

Suction ETT and oropharynx

Suction below vocal cords if possible

Position patient upright

Explain procedure to patient

Have reintubation equipment ready

Procedure:

Deflate cuff

Give positive pressure breath

Remove tube at peak inspiration

Apply oxygen (mask or nasal cannula)

Encourage cough

Post-Extubation:

Monitor closely for stridor

Humidified oxygen

Consider noninvasive ventilation if high-risk

Monitor for respiratory fatigue

# Post-Extubation Management

## Preventing Reintubation

Risk Factors for Extubation Failure:

Prolonged intubation

Weak cough

Excess secretions

Upper airway edema

CHF

Neuromuscular disease

Preventive Strategies:

Noninvasive ventilation (prophylactic in high-risk)

Aggressive pulmonary toilet

Humidification

Early mobilization

Treat underlying causes

Reintubation Criteria:

Respiratory failure

Inability to protect airway

Severe stridor not responding to treatment

Hemodynamic instability

# Case Study 1

## Patient Presentation

Mr. Smith, 65-year-old male

Post-operative day 2 after abdominal surgery

Intubated for respiratory failure

Current settings: VC-AC, VT 500, Rate 14, FIO2 40%, PEEP 5

Current Status:

Peak pressure: 35 cmH2O

Plateau pressure: 28 cmH2O

SpO2: 94%

Patient appears uncomfortable, asynchronous with ventilator

Questions:

Are these pressures acceptable?

What might be causing the asynchrony?

What adjustments would you make?

# Case Study 1 Discussion

## Answers and Rationale

Pressure Assessment:

Plateau 28: Acceptable (<30)

Peak-plateau difference: 7 cmH2O (normal airway resistance)

Pressures are acceptable

Asynchrony Causes:

Pain/anxiety post-surgery

Inadequate sedation

Flow asynchrony (set flow may not match demand)

Possible inadequate tidal volume for patient

Interventions:

Assess pain and sedation needs

Consider pressure support or PC mode for comfort

Check for auto-PEEP

If flow asynchrony suspected, increase flow or switch to PC

Consider spontaneous breathing trial if criteria met

# Case Study 2

## Patient Presentation

Mrs. Johnson, 58-year-old female

ARDS secondary to sepsis

Day 5 of mechanical ventilation

Current settings: VC-AC, VT 400 (6 ml/kg), Rate 22, FIO2 70%, PEEP 12

Current Status:

Plateau pressure: 32 cmH2O

SpO2: 88%

Chest X-ray: Bilateral infiltrates

Questions:

What is the immediate concern?

How would you adjust the ventilator?

What other interventions should be considered?

# Case Study 2 Discussion

## Answers and Rationale

Immediate Concern:

Plateau pressure 32 exceeds safe limit (<30)

Risk of ventilator-induced lung injury

Severe hypoxemia despite high support

Ventilator Adjustments:

Decrease VT to 300-350 ml (4-5 ml/kg)

Check and document driving pressure (Pplat - PEEP)

Increase PEEP per ARDSNet table

Titrate FIO2 to maintain SpO2 88-95%

Other Interventions:

Prone positioning if PaO2/FIO2 <150

Consider neuromuscular blockade if severe

Evaluate for ECMO if refractory

Treat underlying sepsis

Conservative fluid management

# Key Takeaways

Select Mode Based on Patient Needs

Volume control for guaranteed ventilation

Pressure control for comfort

Pressure support for weaning

Always Use Lung-Protective Strategy

6 ml/kg IBW, Pplat <30

Accept permissive hypercapnia if needed

Graphics Tell the Story

Learn to recognize normal and abnormal patterns

Waveforms guide troubleshooting

Treat the Patient, Not the Numbers

Clinical assessment trumps parameters

Comfort and synchrony matter

Wean Early and Daily

Daily SBTs for eligible patients

Use spontaneous awakening trials

# Questions & Discussion

## Contact Information

[Your Name], RRT

[Department]

[Email]

[Phone]

## Additional Resources

ARDSNet Protocol: https://ardsnet.org

AARC Clinical Practice Guidelines

Egan's Fundamentals of Respiratory Care

# References

ARDSNet. Ventilation with lower tidal volumes as compared with traditional tidal volumes for acute lung injury and the acute respiratory distress syndrome. N Engl J Med. 2000;342(18):1301-1308.

AARC Clinical Practice Guidelines: Mechanical Ventilation of Mechanically Ventilated Patients. Respir Care. 2022.

Girard TD, et al. Efficacy and safety of a paired sedation and ventilator weaning protocol for mechanically ventilated patients in intensive care. Lancet. 2008;371(9607):126-134.

Papazian L, et al. Formal guidelines: management of acute respiratory distress syndrome. Ann Intensive Care. 2019;9(1):69.

Tobin MJ. Principles and Practice of Mechanical Ventilation. 4th ed. McGraw-Hill; 2018.

# Thank You

## Respiratory Care Department

Excellence in Mechanical Ventilation Support